# ADXL-202 Accelerometer Servo Leveler Theory

Briefly, the circuit is designed to measure the tilt of the sensor and adjust the position of two servos to move it back to level.

The ADXL-202 outputs pulses varying in width which indicate the amount of plus or minus “g’s” sensed in two axis X and Y.   These two axis can be thought of as fore to aft pitch and side to side tilt.  When level, the X and Y pulse widths are about mid way between the minimum and maximum values.

The platform holding the ADXL-202 is initially leveled and the normally (+5v) SWITCH input to the micro is closed (0v) momentarily, telling the micro to take a measurement of these X and Y pulse widths and store them for future comparisons.

Afterwards, the sensor X and Y outputs are measured against the stored values several times per second.  Any tilt of the sensor is detected as a change in “g’s” and the corresponding axis pulse width changes accordingly.  The micro compares these pulse widths to the known level values stored in memory.   If  there’s any difference, it adjusts the servos in the opposite direction until the measured and stored values match again.

The circuit outputs pulses directly to the pulse inputs of two R/C type servos.  At start up, the circuit positions the servos to their midpoint.  These servo pulses can vary in width between 1 and 2 mS depending on desired position with 1.5 mS being centered.  There is a 20 mS gap between pulses on each servo to allow time for multiple servos.

One important note: use separate power supplies for the servo pulsing circuit and the servos themselves.  Otherwise the electronic noise generated by the servo motors will cause erratic operation of the control circuitry.   Most R/C vehicles do this by using separate batteries for servo power and control power.

© 2004  By Mike Berg